(Platts NWE: PJAAV00)

Annex A

| Specifications Type JET A1 - |  |  |
| --- | --- | --- |

| COMPONENT | UNIT Min/Max | Test Methods ASTM / IP / GOST |
| --- | --- | --- |

| COMPOSITION |  |  |
| --- | --- | --- |
| Appearance | C&B (1) | Visual |
| Colour, Saybolt | Report (2) | D156, D6045 |
| Acidity, Total (mg KOH/g) | Max. 0.10 | D3242, IP 354 |
| Aromatics (vol %) | Max. 25.0 | D1319, IP 156, GOST R 52063 |
| Sulphur, Total (wt %) | Max. 0.25, | D1266, D1552, D2622, D4294, D5453, IP107, IP 243, IP 336, IP 373, IP 447, |
|  |  | GOST R 51947, GOST R 51859 |
| Sulphur, Mercaptan (wt %) | Max. 0.0030 (3) | D3227, IP 342, GOST R 52030 |
| OR Doctor Test | Negative (3) | D4952, IP 30 |
| VOLATILITY |  |  |
| Distillation Temperature: |  | D86, IP 123 |
| 10% Recovery (°C) | Max. 205.0 |  |
| 50% Recovery (°C) | Report |  |
| 90% Recovery (°C) | Report |  |
| Final BP (°C) | Max. 300.0 |  |
| Distillation Residue (vol %) | Max. 1.5 |  |
| Distillation Loss (vol %) | Max. 1.5 |  |
| Flash Point (°C) | Min. 38.0 | D56 (4), D3828, IP170 |
| Density @ 15°C (kg/m3) | 775.0 - 840.0 | D1298, D4052, IP 160, IP 365, GOST R 51069 |
| FLUIDITY Freezing Point (°C) Viscosity @ -20°C (cSt) | Max. -47.0 (5) Max. 8 | D2386, D5972, D 7153, IP 16, IP 435, IP 529 D445, IP 71 |
| COMBUSTION |  |  |
| Net Heat of Combustion (MJ/kg) | Min. 42.80 (6) | D3338, D4529, D4809, IP 12, IP 355 |
| Smoke Point (mm) | Min. 25 | D1322, IP 57 |
| OR Smoke Point (mm) | Min. 19 | D1322, IP57 |
| AND Naphthalenes (vol %) | Max. 3 | D1840 |
| CORROSION Copper Strip (2h @ 100°C) | Max. No.1 | D130, IP 154 |
| THERMAL STABILITY JFTOT 6P @ 260ºC (mm Hg) Tube Rating (Visual) | Max. 25 Max. <3 (7) | D3241, IP 123 |
| CONTAMINANTS |  |  |
| Existent Gum | Max. 7 (8) | D381, IP 131 |
| Water Reaction Interface Rating | Max. 1b | D1094 |
| MSEP Rating Fuel without SDA | Min. 85 |  |
| Fuel with SDA | Min. 70 |  |
| Particulates (mg/dm3) | Max. 1.0 (2) | D5452, IP423 |
| OTHER |  |  |
| Conductivity (pS/m) | 50-600 | D2624, IP 274, GOST 25950 |
| Without SDA | Max. 10 |  |
| BOCLE Wear Scar Diameter (mm) | Max. 0.85 (9) | D5001 |
| ADDITIVES |  |  |
| Antioxidant | Optional (24 mg/L max) |  |
| Static Dissipator | Optional (10) |  |
| Lubricity Improver | Optional (11) |  |

- Fuel should be clear, bright and visually free from solid matter & undissolved water at ambient air temperature.

- Applies at point of manufacture.

- In the event of a conflict between Sulphur Mercaptan and Doctor Test results, Sulphur Mercaptan shall prevail.

- When testing by method D 56, a minimum Flash Point of 40ºC applies.

- Lower freezing point may be agreed between customer & producer.

- When determining the Net Heat of Combustion by ASTM D4529, use equation 1 or Table 1, and when using D3338, use equation 2.

- No peacock or abnormal colour deposits allowed.

- Air can be used instead of steam as the vaporising agent.

- Applies at point of manufacture if fuel contains >95 % hydroprocessed material of which >20% has been severely hydroprocessed.

- Maximum initial doping is 3 mg/L. Upon redoping the fuel, the maximum allowed cumulative concentration is 5 mg/L.

- Hitec 580 may be injected at 15 - 23 mg/L.